# Eco Capsule Shrinker Operations Manual

V1.1

TABLE OF CONTENTS

- SAFETY	Page

  - GENERAL SAFETY	3

  - ELECTRICAL SHOCK	3

  - CONTACT MATERIALS COMPATIBILITY	3

  - SAFETY COMPLIANCE LIABILITY	4

  - CONVENTIONS	4

- MACHINE FEATURES & SPECIFICATIONS

  - INTRODUCTION	5

  - FEATURES & BENEFITS	5

  - UTILITY REQUIREMENTS	5

- INSTALLATION & START-UP

  - INSTALLATION PROCEDURES	6

  - START-UP & COMMISSIONING	6

- MECHANICAL ADJUSTMENTS

  - COMPONENT OVERVIEW	8

  - THEORY OF OPERATION	9

  - CONTAINER GUIDE RAIL AND INDEXING GATE ADJUSTMENTS	10

  - CONTAINER SENSOR ADJUSTMENTS AND LOCATIONS	11

  - FILL BAR ADJUSTMENTS	11

  - FILL HEAD SIDE TO SIDE ADJUSTMENT	12

  - DRIP TRAY HEIGHT ADJUSTMENT	12

  - NECK LOCATOR ADJUSTMENT	13

  - DIVE HEAD AND STROKE ADJUSTMENT	14

- LIQUID HANDLING

  - FLOAT SYSTEM	13

  - SUPPLY pump	14

  - Pressure Pump	14

  - Fill Head	15

- ELECTRICAL

  - FRONT PANEL CONTROLS	16

  - AIR SOLENOIDS	16

- OPERATOR INTERFACE SCREENS

  - MAIN CONTROL SCREEN	17

  - Setup SCREEN 	18

  - ASU SCREEN 	19

  - Timers SCREEN 1	20

  - timers SCREEN 2 	21

  - Manual SCREEN	22

  - I/O SCREEN	23

  - Recipe SCREEN	24

  - LoGIN SCREEN	25

- TROUBLESHOOTING

  - GENERAL TROUBLESHOOTING	26

- SERVICING

  - CLEANING PROCEDURE	27

- FACTORY CONTACT INFORMATION

  - CONTACT APEX DIRECTLY	28

# SAFETY

  - GENERAL SAFETY

Apex Filling Systems, LLC (APEX) manufactures and designs all of its products so they can be operated safely. However the primary responsibility for safety rests with those who use and maintain these products. The following safety precautions are offered as a guide that if conscientiously followed, will minimize the possibility of accidents throughout the useful life of this equipment. The safety of personnel, equipment and plant facilities should be considered during equipment operation and with each changeover of product, or any machine modifications.

Only those who have been trained and delegated to do so and have read and understood this operator's manual should operate the equipment. Failure to follow the instructions, procedures and safety precautions in this manual can result in accidents and injuries.

DO NOT modify the equipment except with written factory approval. Unauthorized equipment modifications will void the warranty.

Each day walk around the equipment and inspect for leaks, loose parts, missing or damaged components, and parts out of adjustment.  Perform all recommended maintenance noted in this manual.

EQUIPMENT SHOULD ALWAYS BE DE-ENERGIZED (POWER AND AIR) BEFORE MAKING MECHANICAL ADJUSTMENTS.

  - ELECTRICAL SHOCK

- To avoid electrical shock hazard, make sure this equipment is properly grounded.

- Dangerous voltages are present within the electrical enclosures. DO NOT operate this equipment with electrical covers open or removed.

- Keep all parts of the body, hand held tools, or other conductive objects away from exposed live-parts of the electrical system. Maintain dry footing and stand on insulating surfaces. DO NOT contact any portion of the equipment when adjusting or making repair to exposed live parts of electrical system.

- Attempt repairs only in a clean, dry, well-lighted, and ventilated area.

  - CONTACT MATERIALS COMPATIBILITY

APEX endeavors to make all contact parts compatible with buyer's products, if known. Because of the wide variety of possible products, Apex Filling Systems, LLC cannot be responsible or liable for ensuring compatibility of contact material with the products. Evaluate material compatibility prior to machine use. Failure to follow this procedure can result in machine damage, fire, operator injury or death

  - SAFETY COMPLIANCE LIABILITY

APEX endeavors to make machinery as safe to operate as possible. National, state and local laws related to safety in the workplace apply primarily to the responsibilities of the employer, and not the equipment manufacturer. The seller agrees to cooperate with the buyer in finding feasible answers to compliance problems. However, because APEX has little control of the many factors which may significantly affect the environment in which this equipment is installed, the seller does not warrant this equipment to be in compliance with OSHA or any like state or local laws or regulations. It is the buyer's responsibility to provide the modifications necessary to assure compliance with the laws and regulations at the point of installation.

A complete inspection of product is necessary until the machinery is proven to produce acceptable results. This should also be performed after every changeover.

  - CONVENTIONS

To ensure the safety of personnel which will install, adjust, maintain and operate this equipment, it is imperative that they understand the dangers, warnings and caution notices. It is important to understand the signal words that may be used throughout this manual.

|  | Alerts to immediate hazard, which will result in death or severe personal injury, if not avoided |
| --- | --- |
|  | Alerts to a hazard which will result in serious injury, or death in some cases, if not avoided. |
|  | Alerts to a potential hazard that may result in a serious personal injury, if not avoided. It also alerts against an unsafe practice that will permanently damage equipment or property. |
|  | Indicates a suggestion as to how to use or adjust the equipment for best product results. |
|  | Points out a proper use that will avoid damage to the equipment, or will extend the life of the parts. |

# MACHINE FEATURES & SPECIFICATIONS

  - INTRODUCTION

APEX fillers are designed to be easy to setup and maintain, capable of providing years of reliable service.  Versatile by design, APEX fillers can accommodate a wide variety of product and container configurations, often without the need for change parts.  Modular design allows for fully automatic or semi-automatic systems.  Tool-less adjustments are available, and allow for easy and quick changeover for various product and container combinations.  Operator controls are easily accessed via the front panel of the unit.

  - FEATURES & BENEFITS

- Easy Changeover

Simple mechanical adjustment for different bottle sizes.  Quick to changeover, simple to use and easy to clean

- Robust

Anodized aluminum and stainless steel shells, frames, legs and housings maximize the working life of your machine, and minimize maintenance costs and downtime

- Customizable

Whatever the production need, APEX has a design to meet

- Flexible

Versatility and Simplicity are intrinsic to the design. Many container sizes and shapes, and many products can be run on one machine

  - UTILITY REQUIREMENTS

Compressed Air Consumption:  Clean, dry (non-oiled) compressed air 90-100psi /

up to 20cfm depending on options

Refer to machine specific documentation.

Electrical Consumption:  110VAC / 60Hz / Single Phase / 20A depending on options

Alternate electrical supply configurations are possible.

Refer to machine specific documentation.

# INSTALLATION & START-UP

  - INSTALLATION PROCEDURES

The assembly should be placed on a solid, level foundation, with the head mount centered over the container conveyor.  The main frame should be leveled using threaded leveling pads or other suitable means to secure the equipment in place.  Electrical connections should be properly terminated into the main electrical enclosure by properly trained technicians, and appropriate supply voltage, proper phase and adequate supply amperage should be verified prior to powering up the equipment.  Sufficient compressed air should be available and connected for proper operation.

  - START-UP & COMMISSIONING

Figure 3-1

Emergency Stop

This manual should be read completely before powering-up the machine. Commissioning of the machine should be performed by a trained technician only after complete understanding of the machine, and with products that match samples indicated to APEX Consultants, LLC if supplied.  After the machine is adjusted for the bottle and product combination, the machine can be put into operation with the following steps.

- Check the machine to see that guards are in place

- Check the mechanical system for loose or missing parts

- Ensure the E-STOP button is depressed, and the unit is clear of personnel.

Figure 3-2

Main Disconnect

- Remove any lock-out/tag-out devices and rotate the main power disconnect clockwise to ON

Figure 3-3

Main Air Filter Regulator

Quick Disconnect Air Inlet

Pressure Adjustment Knob

Pressure Indicator Gauge

Air Filter Particulate and Condensate Trap

Keeping clear of any moving parts and assemblies, remove any lock-out/tag-out devices and rotate the main air disconnect clockwise to ON

- Verify that sufficient containers to be processed are available to be supplied to the machine.

- On the operator interface, press Cycle to begin the indexing.

- If machine does not perform properly, refer to the appropriate mechanical adjustments (Sec 4.1) to rectify, or adjust the program timers (Sec 7.1).

# MECHANICAL ADJUSTMENTs

  - THEORY OF OPERATION

APEX automatic machines utilize a container conveying system, typically a flat tabletop chained conveyor, to move containers into and out of the processing area.  Container location is precisely controlled with the use of guide rails, and indexed using either air cylinders (commonly referred as “gates” or “pins”) to stop and release containers, or using star wheels designed per container, with pockets for container control.

Utilizing timers accessed through the touch screen controls, containers are automatically indexed into the processing area. For pin-indexing setups, an entry gate holds back empty containers, and an exit gate holds back the containers to be processed. The process conveyor will often be stopped while processing containers to increase the stability of the containers.

The cycle starts once the containers have been counted via an electrical container sensor.  Depending upon the equipped options the machine will dive down and encase the capsule for a set amount of time and then retract.

  - CONTAINER GUIDE RAIL AND INDEXING GATE ADJUSTMENTS

Entry Gate

Figure 4-2

Guide Rails and Indexing Gates

Exit Gate

Container Guide Rails

Index Gate Mount Rail

Cylinder Air Flow Control

For containers to move properly down the conveyor, guide rails must be adjusted so that containers move smoothly, without binding, in a uniform manner.  Guide rails will typically be set toward the base of the container for best results.  Difficult-to-control containers may require an alternate setup (multiple rails, special containment, etc) to effectively control the location of the containers.  Guide rails are typically adjusted in/out and up/down with turnbuckles which hold the guide rail rods in place.

Figure 4-3

Indexing Gate Positioning

Figure 4-2  shows a typical (left-to-right) set up of guide rails and indexing gates.  Containers are situated in the fill area by the Entry Gate and Exit Gate locations.  The gates or pins, are mounted in place with L-brackets mounted to a sliding mount rail as shown, and are positioned and locked into place with standard nuts and bolts.

The exit gate is set so that it extends over more than half of the container diameter, so the containers are positively stopped during the indexing cycle.  The entry gate only extends far enough to hold back incoming (empty) containers from moving past it, without affecting their position when extended, as shown in Fig 4-3.

Air cylinders are typically equipped with air flow control valves to adjust the speed which the cylinders will operate.  Turn clockwise to slow the extension or retraction, counter-clockwise to speed.

  - CONTAINER SENSOR ADJUSTMENTS AND LOCATIONS

Figure 4-4

Typical Container Sensor

Sensors are positioned along the container path on the conveyor (such as shown in Fig 4-4) to count and/or verify the container position.  A variety of sensors can be utilized, depending upon the type of container to be sensed (clear glass, plastic bottle, metal can, etc.)  Sensors located prior to the shrink area count the proper number of containers entering the shrink area and verify sufficient containers are available prior to the shrink area. Sensors located downstream from the shrink area will sense if a backup has occurred, and will pause the shrink operation until the backup or jam has clear

# ELECTRICAL

  - FRONT PANEL CONTROLS

Figure 5-4

Front Panel Controls

Typical controls available to the operator are shown in Fig 5-4

The primary operator interface touch screen, Emergency Stop button, Main Power Disconnect, Heater temp and Conveyor Controls are generally mounted on the front panel.

  - AIR SOLENOIDS

Figure 5-5

Air Solenoids

Air solenoids, typically mounted to the side, or rear, of the main control box, provide pneumatic logic control for the machine operation.  An electrical signal, sent from the PLC, activates the solenoid attached to the air valve, switching the air pressure from one port to the other, extending or retracting the connected air cylinder (indexing gates, dive cylinder, drip tray cylinder, etc)

Solenoids may be manually activated by pressing the orange button located on the face of the solenoid assembly to assist with troubleshooting.

# OPERATOR INTERFACE SCREENS

  - MAIN CONTROL SCREEN

Figure 7-1

Main Screen

From the main control screen, the various sub-screens and timers can be accessed.

Pressing the top buttons (in Fig 7-1 “SETUP, MANUAL, TIMERS, and RECIPE”) will access sub-menus described in the following sections:

SETUP:		Section 7.3

TIMERS: 		Section 7.6, 7.7

MANUAL: 		Section 7.8

I/O SCREEN: 		Section 7.9

RECIPE: 		Section 7.10

LOGIN: 		Section 7.11

  - SETUP SCREEN

Figure 7-2

Setup screen

- INDEXING: Sets whether pin indexing will be used for container movement, or if containers will be hand placed or controlled with a method other than standard pin indexing

- LOCATE: Sets whether there is active locating

  - ASU SCREEN 1

ASU Screen handles the indexing ASU

Basic sequence of ASU indexing setup (pin indexing):

    - Ensure the entry and exit gates are properly aligned in the fill area and the guide rails are set properly

    - Ensure the count eye is accurately reading containers

    - Set up sufficient containers to fill the fill area plus any additional containers necessary to reach the count eye

    - The containers should be set up-stream from the entry gate, and the fill area should be clear of containers

    - Press START ASU INDEX to activate the conveyor, the entry gate will open, and the count eye will read the containers

    - When the count eye has read the proper number of containers, the indexing ASU has finished

    - Press SAVE INDEX ASU

  - TIMERS SCREEN 1

Entry Delay: Sets the gap between filled set and new set of containers entering fill zone.

Infeed Count: Number of containers counted each cycle

Count sensor on delay: How long the count sensor needs to be blocked before moving forward

Exit duration: The amount of time needed for containers to move from entry to exit pin.

Heater Dive Delay: The amount of time the heater waits to dive after containers are indexed in.

Heater Shrink Duration: The amount of time the heater stays around the capsule.

  - TIMERS SCREEN 2

Exit retract delay: How long after fill complete until containers begin to index out.

Backup on: How long line back up sensor is blocked before a fault.

Cycle restart delay: How long after fill complete before cycle is restarted. Typically used to slow filler for other equipment down the line.

  - MANUAL SCREEN

The blue buttons act as toggle switches, activating the associated module:

CONVEYOR (AUTO or RUN CONTINUOUS)

Activating this toggle will bypass the conveyor controls, allowing the conveyor to either run for indexing, and stop during filling (AUTO) or to run continuously (RUN)

ENTRY GATE (Enabled or Disabled)

Activating this toggle will bypass the entry cylinder pin/gate controls, allowing the entry cylinder to either open and close for standard pin indexing (Enabled) or to remain open (Disabled)

EXIT GATE (Enabled or Disabled)

Activating this toggle will bypass the exit cylinder pin/gate controls, allowing the exit cylinder to either open and close for standard pin indexing (Enabled) or to remain open (Disabled)

HEATER DIVE

Activating Will Dive the heater

LOCATOR

Activating will extend the locator

7.8 	I/O SCREEN

This screen allows for the operator to view all inputs and outputs for maintenance. 
RECIPE SCREEN

This screen allows the operator to save or load all of the timers (indexing and filling) which are specific to different containers or setups. Pressing directly on the numeral will pop up the numeric entry window. Press the desired address (1-10) and either save or load the data by pressing the associated button

  - LOGIN SCREEN

The log in screen will be used for factory setup. On this screen you may also clear all recipes in a factory reset or to clear a recipe corruption state.

# TROUBLESHOOTING

8.2GENERAL TROUBLESHOOTING

| SYMPTOMS | POSSIBLE RESOLUTIONS |
| --- | --- |
| Machine does not power-up | Verify inlet power is active |
|  | Verify Main Disconnect is rotated to ON |
|  | Check Main Fuses or Circuit Breakers |
|  | Check control transformer (220VAC to 110VAC) and connections |
| Fill levels are inconsistent | Check that float system is properly supplying the hopper tank |
|  | Verify sufficient air pressure is available |
|  | Verify fill nozzles have no clogs, clean if needed |
| Unit powers, but will not cycle | Check that the line backup sensor is located properly, and clear of obstruction |
|  | Check that E-Stop is pulled out |
|  | Check that Reset Power button has been engaged (if equipped) |
| No product in supply tank | Check for proper air supply |
|  | Check pump motor starter relay in control box (if equipped) |
|  | Check supply control valve on supply tank |
|  | Check float switches for proper operation |
|  | Ensure tank drain valve is closed |
|  | Check that product supply is enabled in the Operator Interface |

# SERVICING

CLEANING PROCEDURE

It is important that the machine is kept clean of dirt, broken glass, sand, etc. as these will reduce the wear life of the air cylinders and pump seals, o-rings, etc. The machine should be cleaned with water or soap at regular intervals. Stronger detergents are often used in the food industry and can be corrosive on the machine components. Therefore, the machine should be washed down thoroughly immediately after cleaning with any harsh detergents.

WARNING: When using a high-pressure pistol with cold and hot water, or steam for cleaning, do not spray near any electrical enclosures.

If the machine is to sit dormant for more than 48 hours (or less, depending upon the product) it is good practice to completely flush the product lines, pumps, and fill heads with clean water.  If solvent is needed to dissolve product effectively, it is equally important to flush with clean water after cleaning product residue.  Harsh detergents and solvents can harm seals and tubing and should be cleared from the system.

# FACTORY CONTACT INFORMATION

CONTACT APEX DIRECTLY

Spare Parts

Direct: (219) 402-0211

Service: (219) 363-5463

Apex Filling Systems, LLC

1001 Eastwood Rd

Michigan City, Indiana 46360 USA

(219) 575-7493

www.apexfilling.com

NOTES